November 19, 2021

Sheri Greene

2215 N. 30th St.

Tacoma, WA 98403

Re:	E-21-0406 Civil Comments 1 – LARSON STORAGE PROJECT

Staff has reviewed the above referenced application.  Please incorporate the following comments into the project documentation:

Items to be resolved prior to PERMIT ISSUANCE

Due to significant changes to the prosed development due to zoning and planning regulations this review has been stopped before completion. Some departments have already commented. Please address any comments that are still relevant in the resubmittal.

DOCUMENT REVIEW COMMENTS - STORMWATER REPORT

# Review comments to be provided on second submittal

CIVIL PLAN SHEETS

# Review comments to be provided on second submittal. Some departmental comments are provided:

STORM

# Address inlet and pipe no longer needed as part of frontage improvements on River Rd.

- If inlets are not being used for surface collection, replace/retrofit ring and lid using ROW approved standard.

- Move CB1, on 15th St NW, south to avoid angled connection to existing structure.

# SEWER

# There are two existing laterals that will need to be cut and capped at the main on 15th St NW.  One was for a previously demoed building.  The other is capped behind the sidewalk and unused, it will possibly pose an issue with the storm connection.

- Replace ring and lids within the disturbed areas along River Rd.

- Keep sanitary cleanout at grade for 1526 River Rd.

WATER

# Sheet C1.1 3 of 12:

- There is a water service on 15th St NW that supplies water to 1402, 1404, 1406, and 1408 15th St NW. This service tap needs to be removed from the water main and replaced with a brass plug.  Coordinate with the Water Division for the water main shutdown.  The service tap removals for 1502 River Road (north of this meter) and for the water service directly south of this tap will be done by the Water Division during this same scheduled water main shutdown.

- There are two water services next to each other on River Road northeast of 1526 River Road.  The one to the west is for 1526 River Road, and the one to the east was for 1502 River road, which should no longer be being used.  Since this plan set has a landscape plan, it makes sense that the plants need to be irrigated.  This east water meter could become the irrigation meter.  Protect this service with a DCVA.  Add City Standard detail 03.04.01 to this plan set.

TRAFFIC

# Review comments to be provided on second submittal

STREET

# Review comments to be provided on second submittal

FIRE

- Provide auto-turn or equivalent program to demonstrate fire apparatus turning radius.

- Provide fire lane / no parking signage plan.

- Assuming this storage lot will be protected by fencing? Where would the gates be set? This will limit the access to fire hydrants.

- A stortz fitting will be required on fire hydrant NW 139.

- The size of the storage lot will trigger an onsite fire hydrant. Operationally the SW corner of the storage lot will make it difficult for fire department access with hose lay.

- A fire hydrant will be required onsite in a parking island roughly in the SW corner of the existing building.

PLANNING

# Review comments to be provided on second submittal

Upon incorporating the above comments into the project documents, please return one e-copy of the plans and associated project documentation using the following naming convention:

Permit number,  ReSubmittal Number;  File Name; e.g., E-20-XXXX Resub00X, Civil Plans; E-20-XXXX Resub00X, Landscape Plans; E-20-XXXX Resub00X, Storm Report, etc…

The electronic resubmittal must contain the entire permit resubmittal package including all attachments and a response letter fully responding to all items as outlined above.  All permit resubmittals must be processed through the City’s SharePoint upload file system; please use the link sent to you separately by email or contact the Permit Center for assistance at  253.864.4165, or by email at PermitCenter@puyallupWA.gov.

If you have questions regarding the review comments or revisions please contact me at

253-435-3616 or via email at JCarter@puyallupwa.gov

Sincerely,

Jamie Carter, P.E.

Development Engineer

Cc: File	RBuck